IPR × VLP

Nodal Analysis Desktop Application

PyQt6 GUI Design & Coding Specification

Version 1.0  ·  White + Blue Accent Theme  ·  Production-Ready

1. Project Overview

Build a single-window PyQt6 desktop application for petroleum engineers to perform Nodal Analysis — finding the operating point where the Inflow Performance Relationship (IPR) curve intersects the Vertical Lift Performance (VLP) curve. The UI must be white with blue accents, modern, and engineered-looking.

| Language | Python 3.12+ / PyQt6 ≥ 6.0 |
| --- | --- |
| Charting | pyqtgraph (preferred) or matplotlib embedded via FigureCanvasQTAgg |
| Numeric | NumPy, SciPy (scipy.optimize.brentq for intersection) |
| Theme | Custom QSS stylesheet — white background, blue accents (see §4) |
| Window title | Integrated Production Modelling – Nodal Analysis |
| Min window size | 1280 × 800 px |
| Layout engine | QSplitter (sidebar ¦ chart area) — sidebar fixed 320 px, chart fills remainder |

2. Top-Level Layout

The window is divided into three horizontal bands top-to-bottom, and the centre band splits into two panes left-to-right.

┌─────────────────────────────────────────────────────────────────┐ │  TITLE BAR  ·  App name left  |  Run button right               │ ├──────────────┬──────────────────────────────────────────────────┤ │              │                                                  │ │   SIDEBAR    │            CHART AREA                            │ │   320 px     │   (stretches to fill)                            │ │   scrollable │   pyqtgraph / matplotlib canvas                  │ │              │   toolbar: zoom, pan, export PNG                 │ │   §3.1–§3.5  │                                                  │ ├──────────────┴──────────────────────────────────────────────────┤ │  STATUS BAR  ·  Operating point result or 'No intersection'      │ └─────────────────────────────────────────────────────────────────┘

Use QSplitter(Qt.Orientation.Horizontal) for the middle band. Set sidebar minimumWidth=280 and maximumWidth=380. The chart area has minimumWidth=600. Wrap the sidebar in a QScrollArea so it survives smaller windows.

3. Sidebar — Five Collapsible Sections

Each section is a custom CollapsibleSection widget: a QToolButton header that toggles a QWidget body. Header uses a ▶ / ▼ arrow icon. Collapsed sections show only the header bar. All sections start expanded. A thin horizontal rule (1 px, #90CAF9) separates sections.

3.1  Section 1 — Inflow Performance (IPR)

Header label: "① IPR Model"  |  header accent colour: #1565C0

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| ipr_model_combo | QComboBox | Vogel | Options: Vogel, Fetkovitch, Darcy (linear), Composite Vogel-Darcy |
| Pr | QDoubleSpinBox | 3500.0 | Reservoir pressure · psia · range 0–20 000 · step 50 |
| Pwf | QDoubleSpinBox | 1000.0 | Flowing BHP · psia · range 0–20 000 · step 50 |
| Pb | QDoubleSpinBox | 2800.0 | Bubble-point pressure · psia · range 0–20 000 · step 50. Hidden when model = Darcy |
| Qo_test | QDoubleSpinBox | 500.0 | Test oil rate · STB/day · range 0–50 000 · step 50. Hidden when model = Darcy |
| AOF_label | QLabel (read-only) | — STB/d | Displays calculated absolute open flow. Updated live on any input change |

Live update: connect all spinbox valueChanged signals to a slot that redraws the IPR curve and recalculates AOF without clicking Run.

3.2  Section 2 — Fluid / Oil Properties

Header label: "② Fluid Properties"  |  header accent colour: #1565C0

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| sg_oil | QDoubleSpinBox | 0.82 | Oil specific gravity (water=1). Range 0.60–1.10, decimals 3, step 0.005 |
| sg_gas | QDoubleSpinBox | 0.915 | Gas specific gravity (air=1). Range 0.55–1.50, decimals 3, step 0.005 |
| sg_water | QDoubleSpinBox | 1.07 | Water specific gravity. Range 0.98–1.20, decimals 3, step 0.005 |
| wor | QDoubleSpinBox | 0.0 | Water-oil ratio (STB water / STB oil). Range 0–50, decimals 2, step 0.1 |
| api_label | QLabel (read-only) | — °API | Auto-computed from sg_oil: API = 141.5/sg_oil − 131.5 |

3.3  Section 3 — Wellbore / VLP Data

Header label: "③ VLP — Wellbore"  |  header accent colour: #1565C0

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| tubing_id | QDoubleSpinBox | 0.2034 | Tubing inner diameter · ft · range 0.05–0.50, decimals 4, step 0.001 |
| tubing_od | QDoubleSpinBox | 0.2396 | Tubing outer diameter · ft · range 0.05–0.60, decimals 4, step 0.001 |
| casing_id | QDoubleSpinBox | 0.5042 | Casing inner diameter · ft · range 0.10–1.50, decimals 4, step 0.001 |
| roughness | QDoubleSpinBox | 0.0006 | Absolute pipe roughness · ft · range 0–0.01, decimals 5, step 0.0001 |
| thp | QDoubleSpinBox | 346.6 | Tubing head pressure · psia · range 0–5000, step 10 |
| depth | QDoubleSpinBox | 8244.0 | Total vertical depth · ft · range 100–30 000, step 50 |
| T_surface | QDoubleSpinBox | 80.0 | Surface temperature · °F · range −20–200, step 1 |
| T_bh | QDoubleSpinBox | 130.0 | Bottomhole temperature · °F · range 50–400, step 1 |
| gor | QDoubleSpinBox | 480.0 | Producing GOR · scf/STB · range 0–10 000, step 10 |
| theta | QDoubleSpinBox | 0.0 | Deviation from vertical · degrees · range 0–90, step 1 |

3.4  Section 4 — Rate Range & Step

Header label: "④ Rate Range"  |  header accent colour: #1565C0

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| q_min | QDoubleSpinBox | 50.0 | Minimum liquid rate to sweep · STB/day · range 1–1000, step 10 |
| q_max | QDoubleSpinBox | 3000.0 | Maximum liquid rate to sweep · STB/day · range 100–50 000, step 100 |
| q_step | QDoubleSpinBox | 50.0 | Rate step / resolution · STB/day · range 1–500, step 10. Controls linspace point count |
| dz_step | QDoubleSpinBox | 200.0 | Depth integration step · ft · range 10–1000, step 10. Smaller = more accurate, slower |

Compute: rates = np.linspace(q_min, q_max, int((q_max − q_min) / q_step) + 1)

3.5  Section 5 — Sensitivity Analysis

Header label: "⑤ Sensitivity"  |  header accent colour: #1565C0

Purpose: overlay multiple VLP curves on the same chart to show how one parameter affects the operating point.

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| sens_param | QComboBox | GOR | Parameter to vary: GOR ¦ THP ¦ Depth ¦ WOR ¦ Tubing ID |
| sens_min | QDoubleSpinBox | (auto) | Minimum value of sweep parameter. Pre-filled based on sens_param selection |
| sens_max | QDoubleSpinBox | (auto) | Maximum value of sweep parameter |
| sens_steps | QSpinBox | 4 | Number of curves to draw (including min and max). Range 2–10 |
| sens_run_btn | QPushButton | Run Sensitivity | Triggers sensitivity sweep; each curve is a different colour from a blue→teal palette |
| sens_clear_btn | QPushButton | Clear | Removes sensitivity overlay, restores single VLP curve |

4. Title Bar & Run Button

The top bar is a custom QWidget (height 56 px, background #0D2B55). It contains:

•  App icon (SVG petroleum derrick, 28 px) + label "Nodal Analysis" in white, Inter Bold 16pt — left-aligned

•  A QPushButton labelled "▶  Run Analysis" — right-aligned, background #1565C0, white text, border-radius 6px, height 34px, min-width 140px

•  Clicking Run calls run_analysis() which computes IPR + VLP curves, finds intersection, and updates chart + status bar.

5. Chart Area (Right Pane)

Use a QWidget with a QVBoxLayout. The top half (≈70%) is the plot canvas; below it is a thin legend bar.

5.1  Plot Canvas

Preferred: pyqtgraph PlotWidget. Fallback: matplotlib FigureCanvasQTAgg (NavigationToolbar2QT included).

| Background | #FFFFFF (white) |
| --- | --- |
| Grid | Light grey dashed lines, alpha 0.4 |
| X axis label | Liquid Rate, q (STB/day) |
| Y axis label | Bottomhole Flowing Pressure, Pwf (psia) |
| IPR curve | Solid line, colour #E53935 (red-600), linewidth 2.5, label 'IPR' |
| VLP curve | Solid line, colour #1565C0 (blue-800), linewidth 2.5, label 'VLP' |
| Operating pt | Gold star marker ★, size 14, colour #F9A825, zorder top, labelled 'Operating Point' |
| Sensitivity | Dashed lines, palette: #42A5F5 → #00ACC1 (5-step sequential), linewidth 1.5 |
| Hover tooltip | Show (q, Pwf) coordinates in a floating QLabel or pyqtgraph TextItem on mouse move |
| Interactivity | Zoom: mouse scroll wheel. Pan: left-click drag. Reset: double-click or toolbar button |

5.2  Plot Legend

A QFrame (height 40 px, border-top 1px #C2D6EC) sits below the canvas. It shows colour swatches + labels for all visible curves. Uses a QHBoxLayout with coloured QLabel squares (12×12 px) and text labels.

6. Status Bar (Bottom Band)

A QStatusBar or custom QFrame (height 48 px, background #E3F0FB, border-top 1px solid #90CAF9). Contains a single QLabel that displays one of two states:

| Operating point found | ✅ Operating Point: q* = {value:.1f} STB/day ¦ Pwf* = {value:.1f} psia ¦ Method: Brent bisection |
| --- | --- |
| No intersection found | ⚠️ No Operating Point — IPR and VLP curves do not intersect within the specified rate range. Adjust inputs. |
| Computing | ⏳ Computing… (shown while background thread is running) |
| Error | ❌ Error: {exception message} — check inputs and retry |

Font: Inter Medium 11pt, color #1A2840 for result, #B71C1C for error/no-intersection.

7. ✦ Unique Feature — Live Pressure Traverse Mini-Map

This is the signature element that makes the UI memorable and uniquely useful for petroleum engineers.

In the bottom-right corner of the chart area, embed a miniature Pressure Traverse plot (180×220 px) inside a semi-transparent QFrame (background rgba(255,255,255,0.92), border 1px #90CAF9, border-radius 8px). It shows the wellbore pressure profile (Depth on Y-axis inverted, Pressure on X-axis) at the current operating rate q*.

| Trigger | Redraws automatically whenever a new operating point is found |
| --- | --- |
| Title | "Traverse @ q*" in 9pt Inter, colour #1565C0 |
| Curve colour | #E53935, linewidth 1.5 |
| Y axis | Depth (ft), inverted (0 at top, TVD at bottom) |
| X axis | Pressure (psia) |
| Toggle | Small ⤢ button (12×12 px) in the corner of the mini-map toggles expand/collapse |
| Expand mode | On click, the mini-map animates (QPropertyAnimation, 200 ms) to 360×440 px |

When no operating point exists, the mini-map shows placeholder text: "Run analysis to see traverse."

8. QSS Stylesheet

Apply the following stylesheet to QApplication via app.setStyleSheet(QSS). Adjust hex codes to match the palette.

/* ── Global ─────────────────────────────── */ QWidget { background: #FFFFFF; color: #1A2840; font-family: 'Inter', 'Segoe UI', sans-serif; font-size: 12px; }  /* ── Sidebar scroll area ─────────────────── */ QScrollArea { border: none; background: #F7FAFD; } QScrollBar:vertical { width: 6px; background: #F0F4F8; border-radius: 3px; } QScrollBar::handle:vertical { background: #90CAF9; border-radius: 3px; }  /* ── Collapsible section headers ─────────── */ QToolButton#section_header { background: #E3F0FB; color: #0D2B55; border: none;     border-left: 3px solid #1565C0; padding: 8px 12px; font-weight: 600;     text-align: left; border-radius: 0px; } QToolButton#section_header:hover { background: #BBDEFB; }  /* ── Input widgets ────────────────────────── */ QDoubleSpinBox, QSpinBox, QComboBox {     border: 1px solid #C2D6EC; border-radius: 5px; padding: 4px 8px;     background: #FFFFFF; selection-background-color: #1565C0; } QDoubleSpinBox:focus, QSpinBox:focus, QComboBox:focus {     border: 1.5px solid #1565C0; } QLabel[readOnly='true'] { color: #1565C0; font-weight: 600; }  /* ── Run button ───────────────────────────── */ QPushButton#run_btn { background: #1565C0; color: #FFFFFF; border: none;     border-radius: 6px; padding: 6px 18px; font-weight: 700; font-size: 13px; } QPushButton#run_btn:hover { background: #1976D2; } QPushButton#run_btn:pressed { background: #0D47A1; }  /* ── Sensitivity buttons ─────────────────── */ QPushButton#sens_btn { background: #E3F0FB; color: #1565C0; border: 1px solid #90CAF9;     border-radius: 5px; padding: 5px 12px; font-weight: 600; } QPushButton#sens_btn:hover { background: #BBDEFB; }  /* ── Status bar ───────────────────────────── */ QStatusBar { background: #E3F0FB; color: #1A2840; border-top: 1px solid #90CAF9; font-size: 12px; }

9. Recommended Class Structure

main.py ├── MainWindow(QMainWindow) │   ├── TitleBar(QWidget)            # §4 — navy bar + Run button │   ├── QSplitter(Horizontal) │   │   ├── SidebarWidget(QScrollArea) │   │   │   ├── CollapsibleSection   # reusable; takes title + body QWidget │   │   │   ├── IprSection(QWidget)  # §3.1 — all IPR inputs + AOF label │   │   │   ├── FluidSection(QWidget)# §3.2 — fluid properties │   │   │   ├── VlpSection(QWidget)  # §3.3 — wellbore inputs │   │   │   ├── RateSection(QWidget) # §3.4 — rate range │   │   │   └── SensSection(QWidget) # §3.5 — sensitivity controls │   │   └── ChartWidget(QWidget) │   │       ├── PlotCanvas           # pyqtgraph or matplotlib │   │       ├── LegendBar(QFrame)    # §5.2 │   │       └── MiniTraverseMap(QFrame) # §7 — signature feature │   └── StatusBar(QStatusBar)        # §6 │ ├── engine/ │   ├── pvt.py          # BlackOilPVT class (provided) │   ├── vlp.py          # HagedornBrown class (provided) │   ├── ipr.py          # IPR models: Vogel, Fetkovitch, Darcy │   └── nodal.py        # find_operating_point() via scipy.optimize.brentq │ └── workers/     └── AnalysisWorker(QRunnable)  # runs engine in thread pool; emits signals

10. IPR Models — engine/ipr.py

Implement each model as a function returning Pwf values for an array of rates, plus a companion function for the rate given Pwf.

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| Vogel | Pwf array | — | q/Qmax = 1 − 0.2(Pwf/Pr) − 0.8(Pwf/Pr)² ¦ Qmax from test point |
| Fetkovitch | Pwf array | — | q = C(Pr²−Pwf²)ⁿ ¦ requires Qo_test, Pwf_test; n=0.8 default |
| Darcy (linear) | Pwf array | — | q = J(Pr−Pwf) ¦ J = Qo_test/(Pr−Pwf_test) |
| Composite Vogel-Darcy | Pwf array | — | Darcy above Pb, Vogel below. Requires Pb input |

11. Operating Point Algorithm

Use scipy.optimize.brentq on f(q) = Pwf_IPR(q) − Pwf_VLP(q) over [q_min, q_max].

from scipy.optimize import brentq  def find_operating_point(ipr_fn, vlp_fn, q_min, q_max):     """     ipr_fn(q) -> Pwf_IPR (scalar)     vlp_fn(q) -> Pwf_VLP (scalar, computed from pressure traverse)     Returns (q_star, pwf_star) or raises ValueError if no intersection.     """     f = lambda q: ipr_fn(q) - vlp_fn(q)     # Check sign change exists     if f(q_min) * f(q_max) > 0:         raise ValueError('No sign change — curves do not intersect in range')     q_star = brentq(f, q_min, q_max, xtol=0.1, rtol=1e-5)     pwf_star = ipr_fn(q_star)     return q_star, pwf_star

12. Threading Model

All computation MUST run off the GUI thread to keep the UI responsive. Use QThreadPool + QRunnable with custom Qt signals.

class WorkerSignals(QObject):     result = pyqtSignal(object)   # (q_star, pwf_star, ipr_pts, vlp_pts, traverse_pts)     error  = pyqtSignal(str)     status = pyqtSignal(str)      # progress text  class AnalysisWorker(QRunnable):     def __init__(self, params): ...     def run(self):         try:             # 1. Build IPR curve points             # 2. Build VLP curve points (sweep rates)             # 3. Find operating point             # 4. Compute traverse at q*             self.signals.result.emit(results)         except Exception as e:             self.signals.error.emit(str(e))

13. Python Dependencies (requirements.txt)

PyQt6>=6.5.0 pyqtgraph>=0.13.3 numpy>=1.24 scipy>=1.10 # Optional: matplotlib>=3.7 (fallback charting)

14. Design Token Reference

| Property / Widget | Type | Default | Notes |
| --- | --- | --- | --- |
| --navy | #0D2B55 | — | Title bar, heading text, section number badges |
| --blue | #1565C0 | — | Section header accent, Run button, link colours |
| --blue-hover | #1976D2 | — | Run button hover state |
| --blue-light | #E3F0FB | — | Sidebar background, status bar background |
| --blue-mid | #90CAF9 | — | Divider lines, scrollbar handle, mini-map border |
| --white | #FFFFFF | — | Primary background, input backgrounds |
| --off-white | #F7FAFD | — | Scroll area tint |
| --ink | #1A2840 | — | Body text, axis labels |
| --slate | #4A6080 | — | Secondary text, placeholder text |
| --red | #E53935 | — | IPR curve, error text |
| --gold | #F9A825 | — | Operating point star marker |
| --border | #C2D6EC | — | Input widget borders |

End of Specification

Feed this document directly to any AI coding assistant or use it as your implementation guide.